SECTION 024191 - HISTORIC DEMOLITION

PART 1 - GENERAL

1.1	SECTION INCLUDES

A.	Salvage of existing historic items to be reused or returned to the Owner.

B.	Disposal of removed material and equipment not to be reused or recycled.

1.2	RELATED DOCUMENTS

A.	Demolition notes on drawings: schedule of items/construction to be removed

B.	Structural drawings: Load limits of existing structure, additional demolition and shoring notes.

C.	Drawings of the Original Building: Available at the Office of the Architect and Office of the Owner.

D.	Section 017000 - Execution Requirements

E.	Section 017391 - Historic Work Execution

F.	Section 024100 - Demolition

G.	Section 024291 - Removal & Storage of Historic Materials

H.	Section 011100:    Summary of Hazardous Materials Work

I.	Section 028000 - Asbestos Abatement

J.	Section 028100 - Heavy Metal Related Activities

1.3	ALLOWANCES

A.	See Section 012100 Allowances and the bid form for allowances affecting this Work.

1.4	UNIT PRICES

A.	See Section 012200 Unit prices and the bid form for unit prices affecting this Work.

1.5	DEFINITIONS

A.	Restore in Place: To rework, repair, or repair by replacing missing elements and to then refinish to original appearance and operation

B.	Salvage: To carefully remove and protect an object for either reuse in the project or to deliver to the Owner for other use.

C.	Remove (or Demo or Demolish): To selectively dismantle and remove the material, equipment or structure or entire portion of the Facility(s). These materials are to be disposed of off site by the Contractor.

D.	Disposal: To remove from the site and dispose of in a legal manner.

1.6	SUBMITTALS

A.	Work Plan: See Section 017391

B.	Photo survey: Prior to Demolition, showing building elements to remain. Give special attention to the interface of selective demolition and historic demolition as any damage of historic material to be salvaged is the responsibility of the Contractor to rectify.

1.	See list of Items in Section 024291

C.	Shop Drawings:

1.	Shoring and Bracing Design/Erection Documents

2.	Drawings indicating demolition type, location and construction of temporary construction

3.	Details of the planned condition of all cut surfaces remaining after demolition

4.	Opening protection.

D.	Project Record Documents: Accurately record actual location of capped utilities

1.7	QUALITY ASSURANCE

A.	Demolition Contractor Qualifications:    Company specializing in performing the work of this section with minimum 5 years of experience.

1.	The Contractor shall provide qualified workers trained and experienced in whole-building recycling, including removal and salvage of historic materials.

2.	Any work involving demolition of masonry of any variety shall be performed by worker qualified per Masonry Section involved 042000 or 044301

3.	A list of similar projects shall be provided identifying when, where, and for whom the work was done.

a.	A current point-of-contact for identified references shall be provided.

b.	Demolition Contractor Qualifications:    Per Section 017391

4.	Design shoring and bracing under direct supervision of a Professional Structural Engineer experienced in design of this Work and licensed in the State of Washington.

1.8	REGULATORY REQUIREMENTS

A.	Conform to applicable code for demolition work, dust control, products requiring electrical disconnection and re-connection.

B.	Obtain required permits from authorities.

C.	Do not close or obstruct egress from any building exit or site exit.

D.	Do not disable or disrupt building fire or life safety systems without 3 days' prior written notice to Owner.

E.	Conform to applicable regulatory procedures when hazardous or contaminated materials are discovered.

1.9	REQUIRED WALK THROUGH

A.	All contractors bidding the demolition work are required to attend a walk through of the building.

1.10	PRE-SUBMITTAL MEETING

A.	Convene prior to any submittals required by this section.

B.	Meet under the provisions of Section 013000 Administrative Requirements

1.11	PRE-DEMOLITION MEETING

A.	Convene one week before starting work of this section.

B.	Meet under the provisions of Section Administrative Requirements    "Preconstruction meetings"

1.12	DELIVERY, STORAGE AND HANDLING

A.	Deliver material for temporary protection to project site in manufacturer's original wrapping.

B.	Store above material under cover and elevated above grade.

C.	Protect salvaged material that will be reused.

1.	See Section 024291.

1.13	PROJECT REQUIREMENTS

A.	Conduct demolition to minimize interference with adjacent building elements and areas not subject to demolition.

B.	Cease operations immediately if structure appears to be in danger and notify Architect. Do not resume operations until directed.

C.	Hazardous materials shall be removed per Division 2 section

1.	Remove hazardous materials prior to general demolition work

D.	Owner shall remove loose furnishings, equipment prior to start of demolition work

E.	Architecturally sensitive areas:

Areas included in the Seattle Landmark’s Board Controls and Incentives Agreement are listed below. These areas are designated historic and any modifications to these areas require Landmark’s Board Approval.    Demolition scope of work identified in the permit drawings has been approved by the Seattle Landmark’s Board. Demolition of any element not explicitly stated in the permit drawings must first be reviewed and approved by the architect prior to demolition.

1.	Building Exterior (including the concrete stairwell and the landing on the east elevation)

2.	First floor:

a. Sanctuary, (excluding the pews)

b. Main entrance vestibule

c. The formal stairwells in the side entry vestibules

d. The Pastor’s Room

f. The Organ Pipes and the Pulpit Area

a.	main waiting room

b.	women's waiting room

c.	corridors

d.	compass room

e.	pre-ticketing area

3.	Second floor:

a. The Balcony (excluding the pews)

a.	balcony overlook

b.	corridors

c.	Stair #3 (stairway from 2nd to 3rd floor)

4.	Third floor:

a.	former corridor locations

b.	stairway landing

F.	Any Item scheduled to remain in place or scheduled for salvage that is unnecessarily demolished or lost shall be replaced or rebuilt to resemble the original prior to demolition.

1.14	ENVIRONMENTAL REQUIREMENTS

A.	See 015000 Temporary Facilities and 015721 Indoor Air Quality Controls

1.15	SEQUENCING

A.	Contractor shall coordinate all work in architecturally sensitive areas with restoration trade contractors.(e.g. plaster, wood windows) prior to and throughout demolition work in these areas.

1.16	SCHEDULING

A.	Schedule work under the provisions of Section 013216.

B.	Schedule work to coincide with new construction.

C.	Describe demolition removal procedures and schedule.

D.	Perform noisy work:

1.	As specified in 011000 Summary

PART 2 - PRODUCTS - NOT USED

PART 3 - EXECUTION

3.1	EXAMINATION

A.	Field Verify conditions and boundaries of areas to be demolished prior to the work of this section.

B.	Engineer shall perform a survey of the building prior to the start of demolition work so that procedures for structural support of the building during demolition may be designed.

3.2	PREPARATION

A.	Protect surrounding work both new and existing from damage.

B.	Provide, erect, and maintain temporary barriers at locations indicated and as needed to protect workers, the public and existing work to remain.

C.	Erect and maintain weatherproof closures for existing and new exterior openings.

D.	Erect and maintain temporary partitions to prevent spread of dust, odors, and noise.

E.	Prevent movement of structure; provide bracing and shoring.

F.	Notify affected utility companies before starting work and comply with their requirements.

G.	Notify Architect if conditions or materials encountered are not as described and/or drawn.

H.	Mark location and termination of utilities.

I.	Provide appropriate temporary signage including signage for exit or building egress.

J.	Maintain thorough and strict fire protection protocol during all demolition work.

3.3	PROTECTION

A.	Protect existing or newly created openings in historic material from damage.

1.	As a minimum use the following: 2 inches of EPS rigid insulation under ½ inch plywood.

2.	Submit protection designs per Work Plan: See Section 017391.

3.      Fastening of any temporary protection to the building exterior to occur through masonry unit joints, and not through the face of the masonry.

B.	Protect existing plaster and woodwork to remain, especially at the walls and ceiling of the existing main waiting room, women's waiting room, and compass room Sanctuary, Main Vestibule, formal stairwells in the side entry vestibules, The Pastor’s Room, The Organ Pipes and the Pulpit Area, The Balcony.

C.	Protect existing historic terrazzo used as work surfaces with a minimum 1/2” steel plate over ½” plywood, over 2” EPS high load rigid insulation

* no historic terrazzo in the building

D.	Protect the following materials including any historic materials and equipment to remain in place:

1.	Masonry:

a.	Brick

b.	Stone both interior and exterior as panels and as masonry

c.	Terra Cotta.

2.	Existing waterproofing systems.

3.	Metals:

a.	Copper and other sheet metal decorative elements, gutters, downspouts and flashing.

b.	Metal hand and guard railings.

c.	Cast iron stairs and other cast iron elements.

d.	Metal framing at canopy and similar locations

4.	Wood elements

a.	Wood windows

b.	Wood cabinet work, finish carpentry and trim.

5.	Existing finishes

a.	Plaster

b.	Tile (stone, glass and ceramic)

c.	Terrazzo

d.	Stone cladding, and Stone masonry

6.	See Section 017419 for additional materials and requirements

E.	New and existing openings in the building envelope to remain weatherproof per Section 17000

F.	Protect existing rain water leaders and gutter/downspouts to remain

3.4	DEMOLITION, GENERAL

A.	Comply with Section 017000 - Execution Requirements and 017391 Historic Execution

B.	Comply with Section 024000 – Demolition

C.	Comply with Section 015720 – Archeological Resource Protection

D.	Comply with Section 015721 – Indoor Air Quality Controls

E.	Comply with Section 016116 – Volatile Organic Compound (VOC) content restriction

F.	Comply with Section 017419 – Construction Waste Management and Disposal

3.5	SCHEDULE

A.	See Demolition notes and drawings

3.6	CLEANING

A.	Upon completion of the work, portions of structure to remain and adjacent areas and structures shall be cleaned of dust, dirt, and debris caused by salvage and demolition operations.

END OF SECTION